Australian Capital Territory

# Climate Change and Greenhouse Gas Reduction (Greenhouse Gas Emissions Measurement Method) Determination 2025

## Disallowable instrument DI2025-310

made under the

Climate Change and Greenhouse Gas Reduction Act 2010, s 11 (Measuring greenhouse gas emissions—determinations)

### 1	Name of instrument

This instrument is the Climate Change and Greenhouse Gas Reduction (Greenhouse Gas Emissions Measurement Method) Determination 2025.

### 2	Commencement

This instrument commences on the day after its notification day.

### 3	Determination of method for measuring greenhouse gas emissions

I determine the method for measuring the amount of greenhouse gas emissions in the ACT as set out in schedule 1.

Note	The greenhouse gas emissions measurement method is used, under the Act, s 12 by an independent entity to prepare a report for the Minister about greenhouse gas emissions in the ACT for each financial year.

### 4	Revocation

This instrument revokes the Climate Change and Greenhouse Gas Reduction (Greenhouse Gas Emissions Measurement Method) Determination 2024 (DI2024-311).

Suzanne Orr MLA
Minister for Climate Change, Environment, Energy and Water

24 November 2025

Schedule 1

(see s 3)

- Objects of the determination

This determination sets out the method for the measurement of greenhouse gas emissions arising from sources, or attributable to activities, located within the geographic boundary of the Australian Capital Territory (ACT).

- Application of the determination

The method determined in this instrument must be used to measure the amount of greenhouse gas emissions in the ACT for the year (the annual emissions amount) for inclusion in the annual report prepared by an independent entity as required under section 12 of the Climate Change and Greenhouse Gas Reduction Act 2010 (the Act).

- Greenhouse gas emissions covered

The emissions covered by this determination are:

- Scope 1 emissions from:

- fuel combustion

- fugitive emissions from fuels

- industrial processes

- agriculture

- land use, land use change and forestry

- waste.

- Scope 2 emissions from electricity consumption in the ACT, adjusted for scope 3 electricity transmission and distribution losses.

- Definitions

In this Determination:

carbon dioxide equivalence or CO2-e, means the amount of greenhouse gas multiplied by its specific global warming potential.

dry wood means wood that:

- has a moisture content of 20% or less if the moisture content is calculated on a wet basis; and

- is combusted to produce heat.

emission factors refer to the kilograms of carbon dioxide equivalent emitted per unit of activity.

energy content factor, for a fuel, means gigajoules of energy per unit of the fuel measured as a gross calorific value.

fugitive emissions means the release of emissions that occur during the extraction, processing and delivery of fossil fuels.

global warming potential refers to an index (on a 100 year time horizon) representing the combined effect of the differing times greenhouse gases remain in the atmosphere and their relative effectiveness in absorbing outgoing infrared radiation.

GreenPower means renewable energy purchased in accordance with the Australian Government’s GreenPower program.

Large-scale Generation Certificate (LGC) – see the Renewable Energy (Electricity) Act 2000 (Cwlth), section 5.

scope 1 emissions refer to the emission of greenhouse gases directly resulting from an activity, or series of activities (including ancillary activities).

scope 2 emissions refer to the emission of greenhouse gases that occurs outside the ACT as a consequence of using grid-supplied electricity, heating and/or cooling within the ACT.

scope 3 emissions refer to the emission of greenhouse gases not included as a scope 1 or scope 2 emission that occur outside the ACT as a result of activities within the jurisdiction due to use of goods and services. Scope 3 emissions include electricity transmission and distribution losses.

- Method for calculating emissions from stationary energy

The method for calculating the emissions from stationary energy will be made using the equations presented below:

  - Electricity

The calculation of Scope 2 emissions attributable to consumption of electricity by ACT consumers using the market-based method is particularly complex because of the many different sources of zero emission electricity for which ACT consumers are paying and because of the interaction between the physical and the financial transactions in the National Electricity Market (NEM). Relating to this latter point, the calculation relies on the propositions that the ACT is part of the NSW region of the NEM, that the NSW region exchanges electrical energy with Victoria and Queensland through the relevant interconnectors, that interconnector flows are sourced from the marginal source of generation in each region, and that the marginal source in all three regions is coal fired generation.

In order to make the steps in the calculation somewhat easier to follow, the method is structured in four parts.

The first part calculates the quantity of zero emission electricity (electricity supplied by renewable generators) being paid for by ACT electricity consumers other than renewable electricity being supplied by generators through the ACT Government’s reverse auction processes. This includes the following components:

- the ACT share of national Large Renewable Energy Target scheme generation,

- GreenPower purchases by ACT consumers,

- rooftop photovoltaic (PV) generation from systems under 200 kilowatts (kW), and

- the ACT share of “old” (pre- Renewable Energy Target (RET)) hydro generation (mainly from the Snowy Hydro power stations) forming part of the generation supplying the NSW pool of the NEM.

The second part calculates quantity of electricity considered to be renewable through the voluntary surrender of Large-scale Generation Certificates (LGCs) generated under the ACT Government’s reverse auction process.

The third part calculates residual electricity by subtracting the sum of all renewable electricity (part one and two above) from the total electricity input to the ACT network.

The fourth part calculates emissions by multiplying residual electricity by the national residual mix factor (RMF) for scope 2 emissions as published by the latest Australian National Greenhouse Accounts Factors.

    - Calculate total renewable electrical energy being paid for by ACT electricity consumers

      - Large-scale Renewable Energy Target (LRET) purchases

S1=αi×βi+γi

Where,

S1 = Total LRET purchases (MWh)

αi = Renewable power percentage

βi = Total electricity supplied to residential customers;

γi = Total electricity supplied to non-residential and other customers.

Data sources:

αi - Clean Energy Regulator www.cleanenergyregulator.gov.au/

βi, γi -   Evoenergy (a component of the annual Regulatory Information Notice (RIN) submission to the Australian Energy Regulator (AER))

      - Rooftop PV

S3=iεi

Where,

S3= Total Rooftop PV output (MWh)

iεi = Sum of metered output (MWh) in the year of all PV installations with capacity less than 200 kW in the following categories:

- supplied with ACT feed in tariff (f.i.t.)

- supplied under gross metering but without f.i.t.

- supplied under net metering.

Data sources:

iεi - As advised by Evoenergy in regular reports to the ACT Government, Environment, Planning and Sustainable Development Directorate

      - Below Baseline NSW region NEM renewable generation

S4=m=15Gm5× 1n  x t=1nxi

Where,

1 = Inventory year − 4;

2 = Inventory year − 3;

3 = Inventory year − 2;

4 = Inventory year − 1;

5 = Inventory year.

Where,

n = the number of inventory years from 2012-13 to the current inventory year;

xi = the ACT’s percentage share of below baseline NSW region NEM renewable generation as calculated for the relevant inventory year.

and

For each of the following Stations:  Hume, Blowering, Guthega, Tumut 1, Tumut 2, Tumut 3 (net of pump energy input):

Gm=n=16MinESn,ωn+φn2

Where,

For each of the following Stations:

1 = Hume;

2 = Blowering;

3 = Guthega;

4 = Tumut 1;

5 = Tumut 2;

6 = Tumut 3 (net of pump energy input);

MinESn,ωn+φn2 = the lesser of:

- ESn= electrical energy sent out in the inventory year, and

- ωn+φn2= the simple average of the RET Baseline in the calendar year covering the first half of the reporting year and the calendar year covering the second half of the reporting year.

ED = Electrical energy supplied by TransGrid to Evoenergy;

NDNSW = Total NSW region Native demand;

NGNSW= NSW region Small Non-scheduled Generation;

TLNSW = NSW region transmission losses.

      - Total renewable electrical energy being paid for by ACT electricity consumers

Srenew=n=14Sn

Where,

Srenew = Total renewable electricity being paid for by ACT electricity consumers (MWh)

S1 = Total LRET purchases (MWh)

S2 = Total GreenPower sales in the ACT (MWh)

S3 = Total Rooftop PV output (MWh)

S4 = Below Baseline NSW region NEM renewable generation (MWh)

    - Calculate the renewable electricity associated with the voluntary surrender of LGCs

RElgc = Nlgc

Where,

RElgc  = Renewable electricity associated with voluntary surrender of LGCs (MWh)

Nlgc  = Total number of LGCs voluntarily surrendered by the ACT Government through the Clean Energy Regulator registry (1 LGC = 1 MWh)

    - Calculate the residual electricity

Eres = Etot – Srenew – RElgc

Where,

Eres = Residual electricity (MWh), i.e., from other than identified renewable sources.

Etot = Total electricity input to the ACT network (MWh)

Srenew = Total renewable electricity being paid for by ACT electricity consumers (MWh) (as per 5.1.1.5)

RElgc = Renewable electricity associated with voluntary surrender of LGCs (MWh) (as per 5.1.2)

    - Calculate total Scope 2 emissions attributable to electricity consumed in the ACT

Eelec = Eres x RMF

Where,

Eelec  =  Scope 2 emissions of electricity consumed in the ACT (t CO2-e)

Eres  =  Residual electricity (MWh) (as per 5.1.3)

RMF = Scope 2 Residual Mix Factors (national) (kg CO2-e/kWh)

Data sources:

RMF-  The most recent published edition of the Australian National Greenhouse Accounts Factors

  - Stationary fossil fuel gas (Natural gas) combustion

Annual emissions are calculated using the following equation:

𝐸𝑁G  =  (𝑄𝑁𝐺  -  QTC )  x  𝐸𝐹𝑁𝐺 / 1000

Where:

𝐸𝑁𝐺 is emissions from fossil fuel gas (natural gas) combustion in tonnes of CO2-e.

𝑄𝑁𝐺 is the total amount of fossil fuel gas (natural gas) consumed by end users in the ACT, in gigajoules.

QTC is the amount of fossil fuel gas (natural gas) used by the Transport Canberra bus fleet, in gigajoules.

EFNG is the Scope 1 emission factor for combustion of fossil fuel gas (natural gas) distributed in a pipeline in kilograms of CO2-e per gigajoule.

Data sources:

𝑄𝑁𝐺  -	Evoenergy

QTC  -	ACT Government

EFNG -	The most recent published edition of the Australian National Greenhouse Accounts Factors.

  - LPG stationary combustion

Annual emissions are calculated using the following equation:

𝐸LPG= 𝑄LPG  ×  1.96  ×  EFLPG / 1000

Where:

ELPG is emissions from LPG stationary combustion expressed in tonnes of CO2-e.

QLPG is the consumption of LPG for stationary combustion expressed in tonnes.

1.96 is for converting QLPG from tonnes to kilolitres.

EFLPG is the Scope 1 emissions factor for LPG in kilograms of CO2-e per kilolitre.

Data sources:

QLPG - Total bulk and bottled sales of LPG to ACT consumers; data to be collected from LPG suppliers

EFLPG - The most recent published edition of the Australian National Greenhouse Accounts Factors.

  - Heating oil

EHO=EIW+(QHO×EFHO)/1000

Where:

EHO is emissions from heating oil consumption expressed in tonnes of CO2-e.

EIW is annual Scope 1 emissions in tonnes of CO2-e from combustion of heating oil by Icon Water, as included in the annual report by the business under the National Greenhouse and Energy Reporting Scheme (NGERS).

QHO is the consumption of heating oil in the ACT by users other than Icon Water in kilo litres (if any).

EFHO is the Scope 1 emissions factor for heating oil in kilograms of CO2-e per kilolitre.

Data sources:

EIW- Icon Water

QHO - Total sales of fuel oil to ACT consumers; data to be collected by a survey of users/and/or suppliers.

EFHO - The most recent published edition of the Australian National Greenhouse Accounts Factors.

  - Wood-fuel

Annual emissions are calculated using the following equation:

EWF =  QWF   ×  ECWF  ×  EFWF / 1000

Where:

EWF is emissions from wood fuel consumption expressed in tonnes of CO2-e.

QWF is the consumption of dry wood expressed in tonnes.

ECWF is the energy content factor for dry wood expressed in gigajoules per tonne.

EFWF is the Scope 1 emissions factor for dry wood in kilograms of CO2-e per gigajoule.

Data sources:

QWF - The most recent available ACT government Firewood Sales report

ECWF and EFWF - The most recent published edition of the Australian National Greenhouse Accounts Factors.

  - Fugitive emissions:  Fossil fuel gas (natural gas) distribution

Annual emissions are calculated using the following equation:

Efug  =  UAG  ×  EF  ×  (CCO2   +  CCH4) / 1000

Where:

Efug is the fugitive emissions from the ACT fossil fuel gas distribution network in tonnes CO2-e.

UAG is the quantity of unaccounted for gas in the ACT gas distribution network in the inventory year, derived as the difference between total gas receipts and total network, expressed in gigajoules.

EF is the Emissions Fraction that represents the fraction of gas that is unaccounted for and released as emissions (and not arising from other issues such as measurement error).

CCO2 is the composition factor for CO2 in gas supplied to the ACT, in tonnes CO2-e per terajoule.

CCH4 is the composition factor for methane in gas supplied to the ACT, in tonnes CO2-e per terajoule.

Data sources:

UAG - Evoenergy (to be derived using total gas receipts and total gas network data)

CCO2, CCH4, EF - National Greenhouse and Energy Reporting (Measurement) Determination 2008

- Method for calculating emissions from transport

  - Ground transport

Annual emissions are calculated using the following equation:

𝐸Trans  = ( Σ (𝑄Pi  ×  𝐸CPi  ×  EFPi )  +  (QTC  x 𝐸𝐹𝑁𝐺 )   / 1000

Where:

𝐸Trans is emissions from consumption of road transport fuels in tonnes of CO2-e.

𝑄Pi is the consumption of road transport fuel type i, where i is LPG, petrol, diesel, in kilolitres.

ECPi is the energy content factor for road transport fuel type i, in gigajoules per kilolitre.

EFPi is the Scope 1 emissions factor for road transport fuel type i, in kilograms of CO2-e per gigajoule.

QTC is fossil fuel gas (natural gas) used by the Transport Canberra bus fleet, in gigajoule.

EFNG is the Scope 1 emission factor for fossil fuel gas (natural gas) combustion in kilograms of CO2-e per gigajoule.

Data sources:

𝑄Pi : The Fuel Survey undertaken by the City and Environment Directorate

ECPi , EFPi ,  EFNG : The most recent published edition of the Australian National Greenhouse Accounts Factors.

QTC : ACT Government

  - Aviation

Emissions from aviation are calculated in accordance with the GHG Protocol guidance for cities and account for emissions that occur within the jurisdiction. All emissions associated with the landing and take-off (LTO) cycle (including taxi-out, take-off, climb, descent, land and taxi-in) are taken as a proxy for aviation emissions that occur within the ACT boundary. It is assumed that all cruising altitude emissions occur outside of the ACT. Annual emissions from regular public transport air service movements are calculated using ACERT calculator (ACERT_7.2338_ACI_Public) using the following equation:

Eav=∑LTOa,eFa,e,mEe,m/1000

Where:

Eav  is emissions from fuel combustion in aviation that occurs within the ACT boundary (assumed to be LTO emissions), in tonnes of CO2-e.

LTOa,e  is number of LTO cycles for aircraft type a and engine type e.

Fa,e,m  is fuel consumption per LTO cycle for aircraft type a with engine type e in mode m (taxi out, take-off, climb out, descend, land, taxi-in) [in kilo litres].

Ee,m is emission factor for fuel consumption (in kg CO2-e per kilo litre).

Data sources:

LTOa,e : Canberra Airport

Fa,e,m, Ee,m : Airport Canbon and Emissions Reporting Tool (ACERT_7.2338_ACI_Public)

Emissions from light aircrafts, other than regular public transport, are estimated using AVGAS fuel sales activity data and emissions factors for AVGAS.

- Method for calculating emissions from industrial processes and product use

  - Calculate total product uses as substitutes for ozone depleting substances

    - Commercial and Industrial Refrigerant Emissions are calculated using the following equation:

EACTC&I=ElACTCBElAUSCB×RAUSC&I, AC+GFAACTREFGFAAUSREF×RAUSC&I,REF

Where:

EACTC&I is the emissions resulting from commercial and industrial refrigerant processes in tonnes of CO2-e.

ElACTCB & ElAUSCB is commercial building electricity use in MWh.

RAUSC&I, ACis the national total commercial air conditioning emissions in tonnes of CO2-e.

GFAACTREF & GFAAUSREF is the refrigeration floor area in m².

RAUSC&I,REF is the national total commercial refrigeration emissions in tonnes of CO2-e.

Data sources:

| ElACTCB & ElAUSCB | Commercial Building Baseline Study 2024 – Summary Report (February 2025 Update) |
| --- | --- |
| RAUSC&I, AC | Australian Greenhouse Emissions Information System (AEGIS) |
| GFAACTREF & GFAAUSREF | Commercial Building Baseline Study 2024 – Summary Report (February 2025 Update) |
| RAUSC&I,REF | Australian Greenhouse Emissions Information System (AEGIS) |

    - Residential Refrigerant Emissions are calculated using the following equation:

EACTRES=ElACTSCElAUSSC×RAUSRES, AC+ElACTREFElAUSREF×RAUSRES,REF

Where:

EACTRES is the emissions resulting from residential refrigerant processes in tonnes of CO2-e.

ElACTSC & ElAUSSC is the residential space conditioning electricity use in MWh.

RAUSRES, AC is the national total residential air conditioning emissions in tonnes of CO2-e.

ElACTREF & ElAUSREF is the residential refrigeration electricity use in MWh.

RAUSRES,REF is the national total residential refrigeration emissions in tonnes of CO2-e.

Data sources:

| ElACTSC & ElAUSSC | 2021 Residential Baseline Study for Australia and New Zealand for 2000 to 2040 |
| --- | --- |
| RAUSRES, AC | Australian Greenhouse Emissions Information System (AEGIS) |
| GFAACTREF & GFAAUSREF | 2021 Residential Baseline Study for Australia and New Zealand for 2000 to 2040 |
| RAUSC&I,REF | Australian Greenhouse Emissions Information System (AEGIS) |

    - Mobile Refrigerant Emissions are calculated using the following equation:

EACTVeh=SAUSLVNAUSLV×0.084×NACTLV+SAUSHVNAUSHV×0.108×NACTHV

Where:

EACTVeh is the emissions resulting from mobile refrigerant processes in tonnes of CO2-e.

SAUSLV & SAUSHV is the national stock of refrigerants in light and heavy vehicles in tonnes of CO2-e.

NAUSLV &NAUSHV is the number of light/heavy vehicles at the national level.

NACTLV & NACTHV is the number of light/heavy vehicles in the ACT.

0.084 is the annual leakage rate for light vehicles in tonnes of CO2-e.

0.108 is the annual leakage rate for heavy vehicles in tonnes of CO2-e.

Data sources:

| SAUSLV & SAUSHV | National Inventory Report 2023, Volume 2 (dcceew.gov.au); Table A5.4.1.16 |
| --- | --- |
| NAUSLV &NAUSHV | National Inventory Report 2023, Volume 2 (dcceew.gov.au); Table A5.4.1.16 |
| NACTLV & NACTHV | ACT Government - Total vehicles registered in the ACT |
| 0.084 | National Inventory Report 2023, Volume 2 (dcceew.gov.au); Table A5.4.1.9 |
| 0.108 | National Inventory Report 2023, Volume 2 (dcceew.gov.au); Table A5.4.1.9 |

- Method for calculating emissions from agriculture

Emissions from agriculture will be the sum of emissions from three sources - enteric fermentation, manure management, and agricultural soils. Emissions from enteric fermentation and manure management will be calculated based on the latest available activity data (livestock numbers) reported each year in the Australian National Greenhouse Accounts. The year concerned will normally be one to two years prior to the year for which the ACT inventory is being compiled. Estimates reported for the ACT by the National Greenhouse Gas Inventory will be used for the emissions from agricultural soils.

The following equation will be used to calculate emissions from agriculture:

Eag=l=1nNl×EFefl/1000+l=1nNl×EFmml/1000+Eas

Where:

Eag is emissions from agriculture in tonnes of CO2-e.

Nl is the number of livestock type l in the ACT.

EFefl is the emission factor for enteric fermentation for livestock type l. This is methane in kilograms multiplied by its GWP.

EFmml is the emission factor for manure management for livestock type l. This is methane in kilograms multiplied by its GWP.

Eas is emissions from agricultural soils in tonnes of CO2-e, as reported by the National Greenhouse Gas Inventory for the ACT.

- Method for calculating emissions from land use, land-use change and forestry

ACT Land-use, land-use change and forestry (LULUCF) emissions are the value for the ACT for total emissions from LULUCF, emissions source category 4 under the 2006 IPCC Guidelines, which are contained in the most recent National Greenhouse Accounts compiled by the Department of Climate Change, Energy, the Environment and Water. To date, there has been a time lag in national reporting on annual greenhouse gas inventories, generally of around 18-24 months. To address this, ACT LULUCF emissions for the financial year will be calculated as the average of the three most recent ACT LULUCF emissions results reported in the National Greenhouse Accounts. These data will be updated annually, using the National Greenhouse Accounts to update emissions for reported years and update the three-year rolling average.

- Method for calculating emissions from waste

  - Methane released from landfills

    - Introductory explanation of methodology

The calculations below follow the IPCC (and thus the NGERS) method for estimating emissions from landfills. Various constants and defaults have been used consistent with the current usage in the NGERS method. The output can be achieved by inserting the relevant data into the NGERS solid waste calculator.

The model for decomposition works by creating a record of landfill stock levels of waste in various types for which decomposition is well understood (e.g. food; paper and cardboard; etc.), and then assessing how much of that stock will decompose to create landfill gas in a given year. The overall amount of degradable organic carbon (DOC) is calculated for each waste type as it enters the landfill. The amount of this that subsequently degrades to produce landfill gas is termed decomposable degradable organic carbon (DDOC) and this stock amount is tallied year on year, accounting for degradation, for each waste type.

    - Methodology in detail

Methane released from landfills (other than from flaring of methane) in the inventory year is calculated by the following equation:

Ej  =  [CH4* - γ (Qcap  +  Qflared  +  Qtr)]  ×  (1 – OF)

where:

Ej is the emissions of methane released by the landfill during the year measured in CO2-e tonnes.

CH4* is the estimated quantity of methane in landfill gas generated by the landfill during the year and measured in CO2-e tonnes.

γ  is the factor 6.784 × 10-4 × 25 converting cubic metres of methane at standard conditions to CO2-e tonnes.

Qcap is the quantity of methane in landfill gas captured for combustion from the landfill during the year and measured in cubic metres.

Qflared  is the quantity of methane in landfill gas flared from the landfill during the year and measured in cubic metres.

Qtr is the quantity of methane in landfill gas (if any) transferred out of the landfill during the year and measured in cubic metres.

OF is the oxidation factor (0.1) for near surface methane in the landfill.

The estimation of CH4* takes account of the following factors:

(a) the tonnage of total solid waste disposed of in the landfill in previous years, as set out in Table 1;

(b) the tonnage of total solid waste disposed of in the landfill in the inventory year;

(c) the composition of the solid waste disposed of in the landfill during the year estimated in the categories municipal solid waste (MSW), commercial and industrial waste (C&I), and construction and demolition (C&D) as in Table 1.

(d) the proportions in each of the three categories of the different types of degradable waste, (based on actual data);

(e) the degradable organic carbon content of each of the types of degradable waste disposed of in the landfill by waste type, as set out in Table 2;

(f) the opening stock of degradable organic carbon in the solid waste at the landfill at the start of the first reporting period (financial year 1975) for the landfill is zero;

(g) methane generation constants (k values) for the solid waste at the landfill as per Table 3;

(h) the fraction of degradable organic carbon dissimilated (DOCF) estimated in accordance with Table 4;

(i) the methane correction factor for aerobic decomposition is 1;

The quantity of methane generated by the landfill is calculated by the following equation:

CH4*  = (∆Cost + ∆Cat)  F  1.336  28

where:

CH4* is the quantity of methane generated by the landfill measured in CO2-e tonnes

F is the fraction of methane generated in landfill gas and is 0.5.

1.336 is the factor to convert a mass of carbon to a mass of methane

28 is the 100-year Global Warming potential (GWP) of methane, which converts tonnes of methane to tonnes of carbon dioxide equivalent

∆Cost is the change in the quantity of the opening stock of decomposable degradable organic carbon derived from the sum of all waste mix types located in the landfill during the reporting year, measured in tonnes, lost through decomposition, and estimated by the following equation:

∆Cost = ∑i Cosit  (1-e --ki)

where:

Cosit is the quantity of decomposable degradable organic carbon accumulated in the landfill at the beginning of the reporting year from all waste mix types deposited in all prior years, measured in tonnes and equals:

Cosit = Ccsit-1

where:

Ccsit-1 is the closing stock of decomposable degradable organic carbon accumulated in the landfill in the year immediately preceding the reporting year from all waste mix types defined above, measured in tonnes and equals:

Ccsit = Cosit – ∆Cosit + Cait – ∆Cait

and

ki is the methane generation constant for each waste mix type as specified in Table 3.

and:

∆Cat  is the change in the quantity of decomposable degradable organic carbon derived from the sum of all waste mix types deposited at the landfill during the reporting year, measured in tonnes, lost through decomposition, and equals:

∆Cat = ∑i Cait  [1−e –ki x (13 – M) /12]

where:

Cait is the quantity of degradable organic carbon in all waste mix types deposited at the landfill during the year concerned, measured in tonnes and is equal to:

Cait = (Qit  DOCi  DOCfi  MCF)

where:

Qit is the quantity of each waste mix type deposited at the landfill during the year concerned, measured in tonnes.

DOCi  is the fraction of the degradable organic carbon content in each waste type, as specified in Table 2 of the solid waste for all waste mix types defined above and deposited at the landfill.

DOCfi  is the fraction of decomposable degradable organic carbon for each waste mix types as specified in Table 4.

MCF is the methane correction factor for aerobic decomposition for the facility during the reporting year and is equal to 1.

and where:

i is the waste type.

t is the reporting year.

M is the number of months before commencement of methane generation at the landfill (here zero) plus seven.

Σi is the sum for all waste mix types.

# Table 1: Waste deposition baseline for emissions model

| Financial Year | MSW | C&I | C&D | Total waste |
| --- | --- | --- | --- | --- |
| 1975 | 20,896 | 14,105 | 17,239 | 52,239 |
| 1976 | 26,790 | 18,083 | 22,102 | 66,975 |
| 1977 | 32,677 | 22,057 | 26,959 | 81,693 |
| 1978 | 38,556 | 26,026 | 31,809 | 96,391 |
| 1979 | 44,345 | 29,933 | 36,585 | 110,862 |
| 1980 | 50,421 | 34,034 | 41,597 | 126,052 |
| 1981 | 56,613 | 38,214 | 46,706 | 141,533 |
| 1982 | 63,556 | 42,900 | 52,434 | 158,890 |
| 1983 | 70,901 | 47,858 | 58,494 | 177,254 |
| 1984 | 78,593 | 53,050 | 64,839 | 196,482 |
| 1985 | 86,629 | 58,474 | 71,469 | 216,572 |
| 1986 | 95,424 | 64,411 | 78,725 | 238,560 |
| 1987 | 104,205 | 70,338 | 85,969 | 260,513 |
| 1988 | 113,336 | 76,502 | 93,502 | 283,341 |
| 1989 | 121,752 | 82,182 | 100,445 | 304,379 |
| 1990 | 135,618 | 91,542 | 111,885 | 339,045 |
| 1991 | 125,331 | 84,599 | 103,398 | 313,328 |
| 1992 | 124,123 | 83,783 | 102,402 | 310,308 |
| 1993 | 122,128 | 82,437 | 100,756 | 305,321 |
| 1994 | 166,319 | 112,265 | 137,213 | 415,798 |
| 1995 | 108,822 | 73,455 | 89,778 | 272,054 |
| 1996 | 100,827 | 68,058 | 83,182 | 252,068 |
| 1997 | 95,192 | 64,255 | 78,534 | 237,981 |
| 1998 | 95,890 | 64,726 | 79,110 | 239,726 |
| 1999 | 101,074 | 68,225 | 83,386 | 252,686 |
| 2000 | 103,634 | 69,953 | 85,498 | 259,084 |
| 2001 | 89,690 | 60,541 | 73,994 | 224,225 |
| 2002 | 88,866 | 63,161 | 68,302 | 220,328 |
| 2003 | 84,207 | 62,810 | 60,049 | 207,067 |
| 2004 | 85,440 | 66,685 | 56,265 | 208,390 |
| 2005 | 84,484 | 68,813 | 51,099 | 204,396 |
| 2006 | 80,130 | 67,951 | 44,232 | 192,313 |
| 2007 | 82,919 | 73,047 | 41,459 | 197,425 |
| 2008 | 87,937 | 80,320 | 39,468 | 207,725 |
| 2009 | 88,297 | 98,150 | 27,522 | 213,969 |
| 2010 | 86,753 | 103,058 | 38,895 | 228,706 |
| 2011 | 88,946 | 127,881 | 51,261 | 268,088 |
| 2012 | 89,373 | 139,784 | 88,686 | 317,842 |
| 2013 | 94,863 | 130,859 | 29,370 | 255,092 |
| 2014 | 102,852 | 114,275 | 21,178 | 238,305 |
| 2015 | 111,156 | 111,542 | 21,095 | 243,793 |
| 2016 | 119,701 | 109,007 | 90,601 | 319,310 |
| 2017 | 122,102 | 108,496 | 280,693 | 511,291 |
| 2018 | 118,258 | 117,269 | 105,404 | 340,931 |
| 2019 | 119,039 | 103,278 | 33,959 | 256,276 |
| 2020 | 121,238 | 92,877 | 48,215 | 262,330 |
| 2021 | 133,997 | 98,186 | 24,186 | 256,369 |
| 2022 | 134,011 | 83,404 | 15,117 | 232,532 |
| 2023 | 124,766 | 85,205 | 10,028 | 219,999 |
| 2024 | 125,623 | 50,899 | 11,427 | 187,949 |

# Table 2: Waste mix types DOC values [Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008]

| Waste mix type | Degradable organic carbon value |
| --- | --- |
| Food | 0.15 |
| Paper and cardboard | 0.40 |
| Garden and green | 0.20 |
| Wood and wood waste | 0.43 |
| Textiles | 0.24 |
| Sludge | 0.05 |
| Nappies | 0.24 |
| Rubber and Leather | 0.39 |
| Inert waste | 0.00 |
| Alternative waste treatment residue | 0.08 |

# Table 3: Waste type k values [Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008]

| Waste mix type | k values |
| --- | --- |
| Food | 0.06 |
| Paper and cardboard | 0.04 |
| Garden and Green | 0.05 |
| Wood | 0.02 |
| Textiles | 0.04 |
| Sludge | 0.06 |
| Nappies | 0.04 |
| Rubber and Leather | 0.04 |
| Alternative waste treatment residue | 0.04 |

# Table 4: Fraction of DOC dissimilated (DOCF) [Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008]

| Waste mix type | DOCF value |
| --- | --- |
| Food | 0.84 |
| Paper and cardboard | 0.49 |
| Garden and green | 0.47 |
| Wood | 0.23 |
| Textiles | 0.5 |
| Sludge | 0.5 |
| Nappies | 0.5 |
| Rubber and leather | 0.5 |
| Inert waste | 0.0 |
| Alternative waste treatment residues | 0.5 |

  - Biological treatment of solid waste

Methane and nitrous oxide released from the composting of biomass materials in an un-enclosed composting facility is calculated by the following equation

Ej  = M  ×  EFj- R

where:

EFj is the emission factor for each gas type (j), being methane or nitrous oxide, released from the composting process measured in tonnes of CO2-e per tonne of waste processed, having the following values:

For the gas type methane	0.019

For the gas type nitrous oxide	0.029

Ej is the emissions of the gas type (j), being methane or nitrous oxide, released from the landfill during the year from the composting process measured in CO2-e tonnes.

M is the mass of biomass materials treated by composting during the year measured in tonnes of waste.

R is:

(a)	for the gas type methane—the total amount of methane recovered during the year at the site from the composting of biomass materials measured in tonnes of CO2-e; or

(b)  for the gas type nitrous oxide—zero.

  - Wastewater treatment and discharge

Annual emissions from nitrous oxide and methane emissions arising from wastewater treatment by Icon Water, as included in the annual report by the business under the National Greenhouse and Energy Reporting Scheme (NGERS).

Data source:	Icon Water