Australian Capital Territory

Planning (Exempt Development) Single Dwelling Housing Development Control Declaration 2025 (No 2)

Notifiable instrument NI2025-492

made under the

Planning (Exempt Development) Regulation 2023, s 14 (Territory planning authority may declare development controls)

1	Name of instrument

This instrument is the Planning (Exempt Development) Single Dwelling Housing Development Control Declaration 2025 (No 2).

2	Commencement

This instrument commences on the day after its notification day.

3	Declaration

I declare the development control at schedule 1 for the development of single dwelling housing in residential zones.

4	Revocation

This instrument revokes the Planning (Exempt Development) Single Dwelling Housing Development Control Declaration 2025 (No 1) (NI2025-431).

George Cilliers

Chief Planner

1 September 2025

Residential Zones - Single Dwelling Housing Development Control

Contents

Application	3

1.	Site coverage	4

2.	Plot ratio	4

3.	Pedestrian access	4

4.	Private open space	4

5.	Principal private open space	5

6.	Height of building	5

7.	Number of storeys	5

8.	Building envelope	6

9.	Front boundary setbacks	6

10.	Side and rear boundary setbacks	6

11.	Allowable setback encroachments	7

Tables 1-3 – Front boundary setbacks	7

Table 1: Front boundary setbacks – blocks in subdivisions approved originally before 18 October 1993	7

Table 2: Front boundary setbacks – blocks in subdivisions approved on or after 18 October 1993 but before 31 March 2008	8

Table 3: Front boundary setbacks – blocks in subdivisions approved on or after 31 March 2008	8

Tables 4-7 – Side and rear boundary setbacks	10

Table 4: Side and rear boundary setbacks – large blocks	10

Table 5: Side and rear boundary setbacks – mid sized blocks in subdivisions approved before 2 October 2009	10

Table 6: Side and rear boundary setbacks – mid sized blocks in subdivisions approved on or after 2 October 2009	11

Table 7: Side and rear boundary setbacks – compact blocks	11

12.	Solar building envelope	11

13.	Front fences and walls	12

14.	Courtyard walls	12

15.	Planting area	13

16.	Tree planting	13

Table A: Tree sizes and associated planting requirements	13

17.	Water sensitive urban design	14

18.	Minimisation of cut and fill	15

19.	Road noise	15

19A. Noise affected blocks	15

20.	Number of car parking spaces	15

21.	Dimensions of car parking spaces	15

22.	Access for car parking spaces	16

23.	Location of car parking spaces	16

24.	Basement car parking	16

25.	Garage and carport openings	16

26.	Verge crossings	16

27.	Servicing and infrastructure	16

28.	Mid sized block provisions	16

29.	Surveillance blocks	16

## Application

The Residential Zones – Single Dwelling Housing Development Control enables a single dwelling development, including alterations and extensions, to be exempt from requiring development approval in residential zones.  Where development meets all the relevant requirements set out in this control declared under section 14 (1) (a) of the Planning (Exempt Development) Regulation 2023 (the Regulation), and where it meets the relevant development exemption criteria as set out in the Regulation.  Where a requirement for a specific block is contained within a district single dwelling housing development control for exempt development declarations, that requirement supersedes the corresponding requirement in the Residential Zones – Single Dwelling Housing Development Control.

Where a lawfully constructed single dwelling development does not comply with one or more of the requirements set out in this control, an alteration or extension may still be exempt provided the alteration/extension does not further increase the non-compliance and the development meets all other requirements set out in the control.

This category of exempt development allows compliant single dwellings to be built without development approval.

An exemption declaration is a minor approval that can be issued by the Territory Planning Authority that allows a single dwelling, which would be exempt from development approval other than for one or more minor encroachments, to continue to be dealt with as exempt development.  In all cases, the departures from the requirements in this control must be consistent with the exemptions for minor non-compliance as set out in the Planning (Exempt Development) Regulation 2023 and the Territory Planning Authority is satisfied that the departures are minor and will not cause an adverse effect to neighbours.

Adding a secondary residence or works to a dwelling with a party wall (including demolition) is not exempt development and requires development approval.

Terms used in this development control have the same meaning that they have in the Territory Plan Part G Dictionary and in the Planning (Exempt Development) Regulation 2023.

| Control |  |
| --- | --- |
| ### Site coverage | Site coverage is a maximum of: - For large blocks: 40% of the block area - For mid sized blocks: 60% of the block area - For compact blocks: 70% of the block area. For single dwellings on blocks created through the subdivision of an original residential block, site coverage is a maximum of: - In RZ1 and RZ2: 45% of the block area - In RZ3, RZ4 and RZ5: 50% of the block area. Note: Single dwelling for the purposes of calculating site coverage includes all associated roofed Class 10 buildings. |
| ### Plot ratio | Plot ratio on a large block is a maximum of 50%. |
| ### Pedestrian access | For blocks with a boundary to a rear lane, pedestrian access is provided from the main street frontage. |
| ### Private open space | Minimum private open space for single dwellings is: - For large blocks: - 60% of the block area, less 50m2 - Have a minimum dimension of 6m for an area not less than 10% of the block area. - For mid sized blocks: - 40% of the block area, less 50m2 - Have minimum dimensions as follows: - For blocks less than 360m2 – 5m for an area not less than 10% of the block area - In all other cases – 6m for an area not less than 10% of the block area. - For compact blocks: 30% of the block area Notes: - Principal private open space may be contained within an area of private open space. - Private open space can contain pergola posts, brick columns, retaining walls, raised planter beds, balustrades, courtyard walls, rainwater tanks, clotheslines, hot water systems, domestic fuel tanks, cooling or heating appliances, solar inverters, batteries and the like. Exemption criteria for these structures may be outlined separately in the Planning (Exempt Development) Regulation 2023. - For the purposes of a) ii) and b) ii) – minimum dimension is the minimum length of the shortest dimension for the private open space area/s. |

| ### Principal private open space | At least one area of principal private open space on the block complies with the area and dimensions specified in the table below and meets all of the following: - is located, either: - wholly on the same level as the habitable room mentioned in b), or - at ground level - is located 5m or less from, either: - a habitable room other than a bedroom, or - an outdoor terrace, pergola, patio, deck and the like that is directly adjacent to a habitable room other than a bedroom - is located behind the building line (where the front boundary is to a public road), except where fully enclosed by a courtyard wall - if visible from an adjoining public street or public open space, it is screened by a minimum 1.5m high screen Zone; Block type; Dwelling Size*; Minimum Area; Minimum Dimension**. all; Compact block; all; 16m2; 4m. RZ1 RZ2; Mid sized block Large block; up to 105m2; 28m2; 4m. Mid sized block Large block; 105m2 or greater; 36m2; 6m. RZ3 RZ4 RZ5; Mid sized block Large block; all; 24m2; 4m. * For the purpose of this table, dwelling size is defined as the sum of all floor area measured to the outside face of externals walls including internal walls between the living areas and garage (but excluding the garage). **Minimum dimension relates to minimum length of the shortest dimension of the principal private open space area. Notes: - For the purposes of this control, principal private open space cannot contain swimming pools, fences, retaining walls or courtyard walls. However, exemption criteria for these structures may be outlined separately in the Planning (Exempt Development) Regulation 2023. - For the purposes of d), screening may include courtyard wall, fencing, vegetation or a combination thereof. - Further guidance may also be available as an Advisory Note, available in the Resources section on the Planning website. |
| --- | --- |
| ### Height of building | Maximum height of the building above datum ground level is: - RZ1 and RZ2 – 8.5m - RZ3 – 9.5m - RZ4 and RZ5 – 12.5m Note: maximum height of building for an exempt single dwelling is 12.5m |
| ### Number of storeys | Maximum number of storeys is: - RZ1 and RZ2 –2 storeys. - RZ3 –2 storeys. - RZ4 and RZ5 –3 storeys. |
| ### Building envelope | For mid sized blocks and large blocks, buildings are sited wholly within the building envelope formed by planes projected over the subject block at 45° to the horizontal from a height of 3.5m above each side boundary and rear boundary, except for side boundaries or rear boundaries where solar building envelope requirements apply. This provision does not apply to dwellings on compact blocks. Encroachments outside the building envelope are permitted for flues, chimneys, antennae, aerials, cooling appliances and heating appliances. Notes: - This does not apply to any part of a building that is required to be built to a boundary of the block by a district single dwelling development control. - For the purposes of this control all height measurements are taken from datum ground level. |
| ### Front boundary setbacks | Front boundary setbacks comply with minimum dimensions in Tables 1-3. Notes: - Where a block has more than one street frontage, such as corner blocks, and required front setbacks are not specified in the relevant district single dwelling development control, the secondary street frontage can be nominated by the applicant. The applicant must consider the front setback to existing buildings. - Public open space refers to unleased land that is intended to remain accessible by the public. - On a block with existing buildings, the applicable minimum boundary setbacks are determined by the existing buildings or garage/carport. Existing building setbacks are also used to determine the primary or secondary front boundary. Alterations and extensions are to comply with setbacks in Tables 1-3. - Chamfers may be included in the secondary street frontage, but only if the length of the chamfer is less than the length of the front boundary it is proposed to be added to. |
| ### Side and rear boundary setbacks | Side and rear boundary setbacks: - Comply with minimum dimensions in Tables 4-7.* - For walls less than 900mm from a side boundary and/or rear boundary: - No windows are permitted within any part of the wall. - Garages and carports on large blocks – the maximum length of all walls facing the boundary is 8m. - Dwellings on mid sized blocks –* - the total length of all walls is a maximum of 13m in length - the wall extends no more than 2.5m into the rear zone. * a) and b) iii) do not apply to an integrated housing development parcel where it does not adjoin a residential block that is not in that integrated parcel. Notes: - On a vacant block or a block with no residual buildings, the minimum side boundary setbacks are nominated by the applicant or nominated in a district single dwelling development control. - On a block with existing buildings, the allocation of side boundary 1 and side boundary 2 must consider the side and rear setbacks of existing buildings or garage/carport. Alterations and extensions are to comply with setbacks in Tables 4-7. - For the purposes of b) ii), carports with posts only are not considered walls. - For the purposes of b) iii), dwellings include associated Class 10a structures. - For the purposes of b) iii) A., the total length of all walls for a dwelling on a mid sized block is the sum of all walls less than 900mm from a boundary. |
| ### Allowable setback encroachments | Encroachments into the minimum setback are permitted for: - an eave or roof overhang with a horizontal width of not more than 600mm. - fascias, gutters, downpipes, light fittings and sun blinds. - landings, steps or ramps, none of which are more than 1m above finished ground level. - for side and rear setbacks only - rainwater tanks, clotheslines, hot water systems, chimneys, flues, domestic fuel tanks, cooling or heating appliances, electricity and gas meters, aerials, antennae, unroofed pergolas, solar inverters and batteries. Note: Noise from appliances must comply with the applicable noise standards. |

# Tables 1-3 – Front boundary setbacks

## Table 1: Front boundary setbacks – blocks in subdivisions approved originally before 18 October 1993

|  | block size | primary front boundary setback | exceptions |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | front boundary setback to secondary street frontage | front boundary setback to public open space or pedestrian paths wider than 6m at the widest point | front boundary setbacks to public open space or pedestrian paths of 6m or less at the widest point |
| lower floor level | Large block | 6m | 4m | 4m | 1.5m |
|  | Mid sized block |  | 3m |  |  |
|  | Compact block |  |  |  |  |
| upper floor level | Large block | 6m | 6m | 4m | 1.5m |
|  | Mid sized block |  | 3m |  |  |
|  | Compact block |  |  |  |  |
| garage and/or carport | all | 6m | 5.5m | 4m | 0m |

## Table 2: Front boundary setbacks – blocks in subdivisions approved on or after 18 October 1993 but before 31 March 2008

|  | block size | primary front boundary setback | exceptions |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | front boundary setback to secondary street frontage | front boundary setback to public open space or pedestrian paths wider than 6m at the widest point | front boundary setbacks to rear lane, public open space or pedestrian paths of 6m or less at the widest point |
| lower floor level | Large block | 4m | 4m | 4m | 1.5m |
|  | Mid sized block |  | 3m |  |  |
|  | Compact block |  |  |  |  |
| upper floor level | Large block | 6m | 6m | 4m | 1.5m |
|  | Mid sized block |  | 3m |  |  |
|  | Compact block |  |  |  |  |
| garage and/or carport | all | 5.5m with a minimum of 1.5 m behind the front building line | 5.5m | 4m | 0m |

## Table 3: Front boundary setbacks – blocks in subdivisions approved on or after 31 March 2008

|  | block size | primary front boundary setback | exceptions |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | front boundary setback to secondary street frontage | front boundary setback to public open space or pedestrian paths wider than 6m at the widest point | front boundary setbacks to rear lane, public open space or pedestrian paths of 6m or less at the widest point |
| lower floor level | Large block | 4m | 3m | 4m | 0m |
|  | Mid sized block | 4m* |  | 3m |  |
|  | Compact block | 3m |  |  |  |
| upper floor level | Large block | 6m | 3m | 4m | 0m |
|  | Mid sized block | 4m |  | 3m |  |
|  | Compact block | 3m |  |  |  |
| garage and/or carport | all | 5.5m with a minimum of 1.5m behind the front building line**. |  | 4m | 0m |

*Articulation elements can extend up to 1m into the front setback. Elements can include verandahs, porches, awnings, shade devices, pergolas and the like (a carport is not considered an articulation element)

** the minimum of 1.5m behind the front building line does not apply where a courtyard wall is located in the front zone.

# Tables 4-7 – Side and rear boundary setbacks

## Table 4: Side and rear boundary setbacks – large blocks

|  | minimum side boundary setback within the primary building zone | minimum side boundary setback within the rear zone | minimum rear boundary setback |
| --- | --- | --- | --- |
| lower floor level* | 1.5m | 1.5m | 3m |
| upper floor level – external wall | 3m | 6m | 6m |
| upper floor level – unscreened element | 6m | 6m | 6m |
| Garage* and/or carport | 0m** | 0m** | 3m |

* includes basements

** A 0m setback is only permitted on one boundary

## Table 5: Side and rear boundary setbacks – mid sized blocks in subdivisions approved before 2 October 2009

|  | minimum side boundary setback within the primary building zone |  | minimum side boundary setback within the rear zone |  | minimum rear boundary setback |
| --- | --- | --- | --- | --- | --- |
|  | side boundary 1 | side boundary 2 | side boundary 1 | side boundary 2 |  |
| lower floor level* | 3m | ≥15m frontage 1.5m <15m frontage 0m | 3m | 1.5m | 3m |
| upper floor level – external wall | 3m | 3m | 6m | 6m | 6m |
| upper floor level – unscreened element | 6m | 6m | 6m | 6m | 6m |
| garage* and/or carport | 3m | 0m | 3m | 0m | 3m |

* includes basements

## Table 6: Side and rear boundary setbacks – mid sized blocks in subdivisions approved on or after 2 October 2009

|  | minimum side boundary setback within the primary building zone |  | minimum side boundary setback within the rear zone |  | minimum rear boundary setback |
| --- | --- | --- | --- | --- | --- |
|  | side boundary 1 | side boundary 2 | side boundary 1 | side boundary 2 |  |
| lower floor level* | 1.5m | 0m | 3m | 0.9m | 3m 0m** |
| upper floor level – external wall | 3m | 1.5m 0m** | 6m | 6m | 6m 0m** |
| upper floor level – unscreened element | 6m | 6m | 6m | 6m | 6m |

* includes basements

** only where specifically permitted under a district single dwelling development control

## Table 7: Side and rear boundary setbacks – compact blocks

|  | minimum side boundary setback |  |  | Minimum rear boundary setback |
| --- | --- | --- | --- | --- |
|  | side boundary 1 or longer side boundary of a corner block | side boundary 2 | shorter side boundary of a corner block |  |
| lower floor level* – external wall | 0m | 0m | 3m | 3m 0m** |
| lower floor level – unscreened element | 1.5m | 1.5m | 3m | 3m |
| upper floor level – external wall | 0m*** | 0m*** | 3m | 4m 0m** |
| upper floor level – unscreened element | 1.5m | 1.5m | 3m | 4m |
| garage*and/or carport | 0m | 0m | 0m | 3m 0m** |

* includes basements

** only where specifically permitted under a district single dwelling development control

*** only where the lower floor level is built to the boundary

| ### Solar building envelope | Buildings are sited wholly within the solar building envelope formed by planes projected over the subject block at 31 o to the horizontal from the height of the ‘solar fence’ on any northern boundary of an adjoining residential block. The height of the solar fence is: - For dwellings on large blocks: - In the primary building zone – 2.4m - All other parts of the boundary – 1.8m. - For dwellings on mid sized and compact blocks: - In the primary building zone – 3m - All other parts of the boundary – 2.3m. For the purposes of this control all height measurements are taken from datum ground level. Notes: - This does not apply to those parts of a boundary where the adjacent part of the adjoining residential block comprises only an access driveway (i.e., a “battle-axe handle”). - This does not apply to the part of the building that is specified in a district specific single dwelling housing development control to be built to the boundary. - This does not apply to compact blocks where a building is required to be mandatory two storeys or more and/or built to the boundary, as outlined in a district single dwelling development control. - For the purposes of this control all height measurements are taken from datum ground level |
| --- | --- |
| ### Front fences and walls | Fences or walls are not permitted forward of the building line except where: - previously approved under an estate development plan or subdivision design application. - is permitted in a relevant district single dwelling development control. - it complies with control 14. Courtyard walls. - is exempt development under the Planning Act 2023 or Planning (Exempt Development) Regulation 2023. |
| ### Courtyard walls | Courtyard walls forward of the building line comply with all the following: - total length complies with one of the following: - not more than 50% of the width of the block - not more than 70% where the width of the block at the line of the wall is less than 12m. - a 50% minimum front setback as specified in Tables 1-3. - a minimum height of 1.5m and a maximum height of 1.8m above datum ground level. - constructed of brick, block or stonework or rendered material, any of which may be combined with timber or metal panels that include openings not less than 25% of the surface area of the panel and clearly distinguishes itself from a panel or timber fence. - incorporate shrub planting along the length of the wall between the wall and the front boundary. - do not obstruct sight lines for vehicles and pedestrians on public paths on driveways in accordance with Australian Standard AS2890.1- Off-Street Parking. |
| ### Planting area | Planting area achieves the following minimum area. To be included in planting area, the area must have a minimum dimension of 2.5m. Block; % of block area. Large block; 30%. Mid sized block; 20%. Compact block; 15%. Notes: - Structures, such as retaining walls, rainwater tanks, hot water systems heating and cooling appliances, are not to be within the 2.5m area. - Artificial turf is not permitted within the planting area |
| ### Tree planting | Development provides a minimum level of tree planting in deep soil zones associated with the requirements in Table A, consistent with the following: - For compact blocks, at least one small tree. - For mid sized blocks, at least two small trees. - For large blocks less than or equal to 800m2, at least one small tree and one medium tree (or equivalent existing tree/s – see Table B). - For large blocks more than 800m2, at least one medium tree and one large tree (or equivalent existing tree/s – see Table B); and one additional large tree or two additional medium trees for each additional 800m2 block area (or equivalent existing tree/s – see Table B) All new trees proposed are planted in accordance with utility services requirements. For existing trees on the site, Table B provides tree size equivalents. |
| ## Table A: Tree sizes and associated planting requirements Tree size; Mature height; Minimum canopy diameter***; Minimum soil surface area dimension; Minimum pot size (litres)*; Minimum soil volume. Small Tree; 5-8m; 4m; 3m; 45**; 18m3. Medium Tree; 8-12m; 6m; 5m; 75**; 42m3. Large Tree; >12m; 8m; 7m; 75**; 85m3. Notes: For the purposes of this table, a tree is defined as a woody perennial plant suitable for the Canberra climate. Any new trees cannot be a plant described in schedule 1 of the Pest Plants and Animals (Pest Plants) Declaration 2015 (No 1) or any subsequent declaration made under section 7 of the Pest Plants and Animals Act 2005, unless the tree is included on the ACT tree register. *Minimum pot size refers to the container size of new trees prior to planting. **The maximum pot size for small, medium and large eucalyptus sp. trees if selected is 45 litres, with maximum height at planting of 2.5m and maximum trunk caliper of 3cm. ***Provided the minimum canopy diameter of the respective tree size can be met, this can be counted as meeting the tree size requirement.. Table B: Tree sizes – equivalents for existing trees Tree size; Tree sizes - Equivalent. Small Tree; An existing tree of a larger size category can also substitute for a planting requirement for a smaller tree.. Medium Tree; 2 small existing trees; or 1 large existing tree. Large Tree; 4 existing small trees; or 2 existing medium trees; or 1 existing medium tree plus 2 existing small trees. |
| ### Water sensitive urban design | Option A All new dwellings, extensions and alterations (except extensions of a size 50% or less of existing gross floor area, or development where no new plumbing is proposed), meet one of the following options: - on compact blocks: - no minimum water storage requirement - minimum WELS rated plumbing fixtures. - on mid sized blocks: - minimum on-site water storage of water from roof harvesting is 2,000 litres - 50% or 75m2 of roof plan area, whichever is the lesser, is connected to the tank - the tank is connected to at least 1 toilet, laundry cold water and all external taps that are attached to the dwelling. The connection will require a pump where it cannot be elevated sufficiently to give adequate pressure. - on large blocks up to 800m2: - minimum on-site water storage of water from roof harvesting is 4,000 litres - 50% or 100m2 of roof plan area, whichever is the lesser, is connected to the tank - the tank is connected to at least 1 toilet, laundry cold water and all external taps that are attached to the dwelling. The connection will require a pump where it cannot be elevated sufficiently to give adequate pressure. - on large blocks greater than 800m2: - minimum on site water storage of water from roof harvesting is 5,000 litres - 50% or 125m2 of roof plan area, whichever is the lesser, is connected to the tank - the tank is connected to at least 1 toilet, laundry cold water and all external taps that are attached to the dwelling. The connection will require a pump where it cannot be elevated sufficiently to give adequate pressure. - Option B: - A greywater system capturing all bathroom and laundry greywater and treating it to Class A standard. The treated greywater is connected to all laundry cold water, toilet flushing and all external taps. - Option C: - Evidence is provided that the development achieves a minimum 40% reduction in mains water consumption compared to an equivalent development constructed in 2003, using the on-line assessment tool or another tool. The 40% target is met without any reliance on landscaping measures to reduce consumption. - Note: The Single residential waterways calculator can be found at: https://www.planning.act.gov.au/professionals/regulation-and-responsibilities/responsibilities/water-efficiency |
| ### Minimisation of cut and fill | The total change in ground level resulting from cut or fill does not exceed 1.5m within 1.5m of a side boundary and/or rear boundary. This does not include a cut associated with a basement. Notes: - The change in ground level is the cumulative total of all level changes within 1.5m of the boundary taken from the datum ground level (DGL) to the new finished ground level (FGL). - Exemption criteria for retaining walls is outlined separately in the Planning (Exempt Development) Regulation 2023. |
| ### Road noise | This control applies to new dwellings and extensions and alterations that add a habitable room located adjacent to an arterial road carrying or forecast to carry traffic volumes greater than 12,000 vehicles per day. The dwelling or the extension to the dwelling is designed and constructed to comply with (or future equivalents) AS/NZS 3671 – Acoustics – Road Traffic Noise Intrusion Building Siting and Design. Note: - Roads carrying 12,000 vehicles per day are generally arterial roads. Classification of roads within the ACT are shown on the ACT Government Active Travel Planning Tool. The City and Environment Directorate (or future equivalent) can be contacted to advise which roads carry or are forecast to carry 12,000 vehicles per day. - Only applies if extension and/or alteration is closer to the road than the existing dwelling |
| ### 19A. Noise affected blocks | This control applies to a block identified as being potentially affected in a district single dwelling housing development control. The dwelling or the extension to the dwelling is designed and constructed to comply with (or future equivalents) AS/NZS 2107.2000 – Acoustics – Recommended design sound levels and reverberation times for building interiors (the relevant satisfactory recommended interior design sound level). |
| ### Number of car parking spaces | A minimum of 2 car parking spaces are provided on the block, unless the development is a single bedroom dwelling on a compact block, in which case a minimum of 1 car parking space is provided. |
| ### Dimensions of car parking spaces | Dimensions of car parking spaces are not less than the following, or as otherwise outlined in AS 2890.1 Off-Street Parking Facilities: - single roofed space - 6m deep x 3m wide - double roofed space - 6m deep x 5.5m wide - single unroofed space - 5.5m deep x 3m wide - multiple unroofed spaces side by side - 5.5m deep x 2.6m wide - parallel parking spaces - 6.7m deep x 2.3m wide - 2.1m minimum clearance to any overhead structure. Note: Dimensions for roofed spaces are internal dimensions. |
| ### Access for car parking spaces | Access for car parking spaces, driveways and vehicle manoeuvring areas complies with relevant requirements in AS 2890.1, the Australian Standard for Parking Facilities, such as manoeuvring to and from and within the development, sightlines and gradients. |
| ### Location of car parking spaces | Car parking spaces are provided to meet the following: - are not located in the front zone; except on: - compact blocks - any part of a driveway in tandem with another car parking space that is located behind the front building line - one car space is roofed - can be in tandem - do not encroach minimum block boundaries. |
| ### Basement car parking | For basement car parking, the maximum total width of an entry and/or exit facing the street is 8m. |
| ### Garage and carport openings | The maximum total width of garage door openings and external width of carports facing a street is 50% of the total length of the building façade facing that street. This does not apply to frontages to rear lanes. |
| ### Verge crossings | Verge crossings comply with the following: - At least one verge crossing per block is provided. - No more than two verge crossings per block are permitted. - All verge crossings comply with applicable design standards and approval requirements - Redundant driveway verge crossings are removed, and the verge and kerb restored. Note: All verge crossings are endorsed by the City and Environment Directorate (or future equivalent), including adding a new driveway, modifying an existing driveway, or adding a second driveway. |
| ### Servicing and infrastructure | Proposed development can be sufficiently serviced in terms of infrastructure and utility services. Endorsement is obtained from relevant utility providers (electricity, water, gas, sewerage and stormwater) to confirm that the location and nature of earthworks, utility connections, proposed buildings, pavements and landscape features comply with utility standards, access provisions and asset clearance zones. |
| ### Mid sized block provisions | Where identified as a mid sized block in a district single dwelling housing development control, the provisions in this control relating to mid sized blocks apply to blocks from 500m2 or greater but less than 550m2 in size. |
| ### Surveillance blocks | Where identified in a district single dwelling housing development control as a surveillance block, dwellings provide habitable room(s) above the garage with windows facing and overlooking the rear lane, or as otherwise determined in a district single dwelling housing development control. |