Australian Capital Territory

Work Health and Safety Amendment Regulation 2024 (No 1)

Subordinate Law SL2024-8

The Australian Capital Territory Executive makes the following regulation under the Work Health and Safety Act 2011.

Dated 26 June 2024.

Andrew Barr

Chief Minister

Mick Gentleman

Minister

Australian Capital Territory

Work Health and Safety Amendment Regulation 2024 (No 1)

Subordinate Law SL2024-8

made under the

Work Health and Safety Act 2011

Contents

Page

1	Name of regulation	1

2	Commencement	1

3	Legislation amended	1

4	Section 9A	1

5	Section 291 (1), definition of high risk construction work, paragraph (s)	3

6	Section 291 (2)	3

7	New part 7A.1 heading	4

8	Section 418A (1), new definition of crystalline silica	4

9	Section 418A (1), definitions of crystalline silica control measure and crystalline silica material	4

10	Section 418A (1), definition of cut	6

11	Section 418A (1), definition of engineered stone	6

12	Section 418A (1), new definition of mechanical method	7

13	Section 418A (1), definition of mechanical process	7

14	Section 418A (1), new definitions	7

15	Section 418A (1), definition of respiratory protective equipment, paragraph (b) and note	7

16	Section 418A (1), new definitions	8

17	Section 418A (3), definition of crystalline silica process, paragraph (a)	8

18	New part 7A.2 heading	8

19	Section 418B heading	9

20	Section 418B	9

21	Section 418B (a)	9

22	Section 418BAA heading	9

23	Section 418BAA	10

24	Section 418C heading	10

25	Section 418C	10

26	Section 418C (b) (i)	11

27	Section 418C (b) (iii)	11

28	Section 418CAA heading	11

29	Section 418CAA (1)	12

30	Section 418CAA (1) (b)	12

31	New part 7A.3	12

32	New division 7A.3.3	15

33	New division 11.2.5	17

34	Dictionary, new definition of crystalline silica	17

35	Dictionary, definition of cut	18

36	Dictionary, new definition of mechanical method	18

37	Dictionary, definition of mechanical process	18

38	Dictionary, new definitions	18

39	Further amendments, dictionary, definitions	19

Schedule 1	Other amendments	20

1	Name of regulation

This regulation is the Work Health and Safety Amendment Regulation 2024 (No 1).

2	Commencement

(1)	This regulation (other than section 32 and schedule 1) commences on 1 July 2024.

Note	The naming and commencement provisions automatically commence on the notification day (see Legislation Act, s 75 (1)).

(2)	Section 32 commences on 1 November 2024.

(3)	Schedule 1 commences on 19 August 2024.

3	Legislation amended

This regulation amends the Work Health and Safety Regulation 2011.

4	Section 9A

substitute

9A	Meaning of corresponding WHS law—Act, dict

For the Act, dictionary, definition of corresponding WHS law, the following laws are prescribed:

(a)	the following laws of the Commonwealth:

(i)	the Work Health and Safety Act 2011;

Note	A reference to an Act includes a reference to any regulation or statutory instrument made or in force under the Act (see Legislation Act, s 104).

(ii)	the Occupational Health and Safety Act 1991 (repealed);

Note	A reference to a repealed Act includes a reference to any regulation or statutory instrument made under that Act (see Legislation Act, s 104).

(iii)	any other law relating to work health and safety matters;

(b)	the following laws of NSW:

(i)	the Work Health and Safety Act 2011;

(ii)	the Occupational Health and Safety Act 2000 (repealed);

(iii)	any other law relating to work health and safety matters;

(c)	the following laws of Victoria:

(i)	the Occupational Health and Safety Act 2004;

(ii)	the Occupational Health and Safety Act 1985 (repealed);

(iii)	any other law relating to work health and safety matters;

(d)	the following laws of Queensland:

(i)	the Work Health and Safety Act 2011;

(ii)	the Workplace Health and Safety Act 1995 (repealed);

(iii)	any other law relating to work health and safety matters;

(e)	the following laws of Western Australia:

(i)	the Work Health and Safety Act 2020;

(ii)	the Industrial Relations Act 1979;

(iii)	the Occupational Safety and Health Act 1984 (repealed);

(iv)	any other law relating to work health and safety matters;

(f)	the following laws of South Australia:

(i)	the Work Health and Safety Act 2012;

(ii)	the Occupational Health, Safety and Welfare Act 1986 (repealed);

(iii)	any other law relating to work health and safety matters;

(g)	the following laws of Tasmania:

(i)	the Work Health and Safety Act 2012;

(ii)	the Workplace Health and Safety Act 1995 (repealed);

(iii)	any other law relating to work health and safety matters;

(h)	the following laws of the Northern Territory:

(i)	the Work Health and Safety (National Uniform Legislation) Act 2011;

(ii)	the Workplace Health and Safety Act 2007 (repealed);

(iii)	any other law relating to work health and safety matters.

5	Section 291 (1), definition of high risk construction work, paragraph (s)

substitute

(s)	involves processing crystalline silica material using a power tool or another mechanical method.

6	Section 291 (2)

substitute

(2)	In this section:

crystalline silica material—see section 418A (1).

light rail—see the Road Transport (General) Act 1999, dictionary.

mechanical method—see section 418A (1).

process—see section 418A (1).

7	New part 7A.1 heading

insert

Part 7A.1	Preliminary

8	Section 418A (1), new definition of crystalline silica

insert

crystalline silica—

(a)	means crystalline polymorphs of silica; and

(b)	includes the following substances:

(i)	cristobalite;

(ii)	quartz;

(iii)	tridymite;

(iv)	tripoli.

9	Section 418A (1), definitions of crystalline silica control measure and crystalline silica material

substitute

crystalline silica control measure—each of the following control measures is a crystalline silica control measure in relation to the processing of engineered stone, stone-substitute material or other crystalline silica material:

(a)	a water delivery system supplying a continuous feed of water over the processing area is used to suppress airborne crystalline silica produced by the processing;

(b)	a wet dust suppression method;

(c)	the attachment of a Class H vacuum to the tool used for processing;

(d)	for other crystalline silica material containing less than 25% crystalline silica—the attachment of a Class M vacuum to the tool used for processing;

(e)	the use of a local exhaust ventilation system;

(f)	the isolation of the place where the processing occurs from other workers.

crystalline silica material means—

(a)	engineered stone; or

(b)	natural stone containing crystalline silica; or

(c)	any of the following containing crystalline silica:

(i)	a concrete or cement product;

(ii)	a brick, paver or other similar block;

(iii)	a ceramic wall or floor tile;

(iv)	grout, mortar or render;

(v)	plasterboard;

(vi)	a porcelain product;

(vii)	sintered stone;

(viii)	a roof tile.

Note	An item mentioned in par (b) or par (c) containing crystalline silica and other materials may be engineered stone (see def engineered stone).

10	Section 418A (1), definition of cut

omit

11	Section 418A (1), definition of engineered stone

substitute

engineered stone—

(a)	means an artificial product that—

(i)	contains 1% or more crystalline silica, determined as a weight/weight (w/w) concentration; and

(ii)	is created by combining natural stone with other materials; and

Examples—other materials

pigment, resin, water

(iii)	becomes hardened; but

(b)	does not include any of the following:

(i)	a concrete or cement product;

(ii)	a brick, paver or other similar block;

(iii)	a ceramic wall or floor tile;

(iv)	grout, mortar or render;

(v)	plasterboard;

(vi)	a porcelain product;

(vii)	sintered stone;

(viii)	a roof tile.

12	Section 418A (1), new definition of mechanical method

insert

mechanical method does not include a method that involves plant or a tool that—

(a)	relies exclusively on manual power for its operation; and

(b)	is designed to be primarily supported by hand.

13	Section 418A (1), definition of mechanical process

omit

14	Section 418A (1), new definitions

insert

porcelain product does not include a porcelain product containing resin.

process includes crush, cut, drill, grind, polish, sand or trim.

15	Section 418A (1), definition of respiratory protective equipment, paragraph (b) and note

substitute

(b)	complies with—

(i)	AS/NZS 1716:2012 (Respiratory protective devices); and

(ii)	AS/NZS 1715:2009 (Selection, use and maintenance of respiratory protective equipment).

Note	AS/NZS 1716:2012 and AS/NZS 1715:2009 do not need to be notified under the Legislation Act because s 47 (5) does not apply (see s 15 and Legislation Act, s 47 (7)). The standards may be purchased at www.standards.org.au.

16	Section 418A (1), new definitions

insert

sintered stone does not include a sintered stone product containing resin.

stone-substitute material means—

(a)	engineered stone; or

(b)	either of the following containing crystalline silica:

(i)	a porcelain product;

(ii)	sintered stone.

17	Section 418A (3), definition of crystalline silica process, paragraph (a)

omit

mechanical process

substitute

mechanical method

18	New part 7A.2 heading

after section 418A, insert

Part 7A.2	General controls on work involving crystalline silica material

19	Section 418B heading

substitute

418B	Dry processing of stone-substitute material—prohibition

20	Section 418B

omit

direct or allow a worker to cut engineered stone with a power tool or use another mechanical process to cut

substitute

process, or direct or allow a worker to process, stone-substitute material with a power tool or by using another mechanical method to process

21	Section 418B (a)

omit

cutting

substitute

processing

22	Section 418BAA heading

substitute

418BAA	Uncontrolled processing of other crystalline silica material—prohibition

23	Section 418BAA

omit

direct or allow a worker to cut crystalline silica material other than engineered stone with a power tool or use another mechanical process to cut

substitute

process, or direct or allow a worker to process, crystalline silica material other than stone-substitute material with a power tool or by using another mechanical method to process

24	Section 418C heading

substitute

418C	Control measures for processing stone-substitute material

25	Section 418C

omit

cutting engineered stone with a power tool or using another mechanical process to cut

substitute

processing stone-substitute material with a power tool or by using another mechanical method to process

26	Section 418C (b) (i)

omit

cutting

substitute

processing

27	Section 418C (b) (iii)

substitute

(iii)	ensuring each worker at the workplace who may be exposed to airborne crystalline silica produced by the processing—

(A)	is provided with respiratory protective equipment; and

(B)	wears the respiratory protective equipment while the processing is carried out.

28	Section 418CAA heading

substitute

418CAA	Control measures for processing other crystalline silica material

29	Section 418CAA (1)

omit

cutting crystalline silica material other than engineered stone with a power tool or using another mechanical process to cut

substitute

processing crystalline silica material other than stone-substitute material with a power tool or by using another mechanical method to process

30	Section 418CAA (1) (b)

omit

cutting

substitute

processing

31	New part 7A.3

insert

Part 7A.3	Engineered stone

Division 7A.3.1	Preliminary

418E	Meaning of processing—pt 7A.3

In this part:

processing, in relation to engineered stone, means using a power tool or other mechanical method to process the stone, and includes crushing, cutting, drilling, grinding, polishing, sanding or trimming the stone.

Division 7A.3.2	Work involving engineered stone benchtops, panels or slabs

418F	Work involving engineered stone benchtops, panels or slabs—prohibition

A person conducting a business or undertaking must not carry out, or direct or allow a worker to carry out, work that involves manufacturing, supplying, processing or installing engineered stone benchtops, panels or slabs.

Maximum penalty:

(a)	in the case of an individual—$8 400; or

(b)	in the case of a body corporate—$42 000.

Note	Strict liability applies to each physical element of each offence under this regulation, unless otherwise stated (see s 6A).

418G	Work involving engineered stone benchtops, panels or slabs—exception for particular supply and installation

Section 418F does not apply to work that involves supplying or installing engineered stone benchtops, panels or slabs if the work is carried out—

(a)	for genuine research and analysis; or

(b)	to sample and identify engineered stone.

418H	Work involving engineered stone benchtops, panels or slabs—exception for particular processing

(1)	Section 418F does not apply to work that involves processing engineered stone benchtops, panels or slabs if the work—

(a)	is carried out—

(i)	for genuine research and analysis; or

(ii)	to sample and identify engineered stone; or

(iii)	to remove, repair or make minor modifications to engineered stone installed—

(A)	before 1 July 2024; or

(B)	in circumstances mentioned in section 418G; or

(iv)	to dispose of engineered stone, whether it is installed or not; and

(b)	is controlled.

(2)	In this section:

controlled—work involving processing engineered stone benchtops, panels or slabs is controlled if any risk associated with the work is—

(a)	eliminated so far as is reasonably practicable; or

(b)	if it is not reasonably practicable to eliminate the risk—minimised so far as is reasonably practicable by—

(i)	using a water delivery system supplying a continuous feed of water over the processing area to suppress airborne crystalline silica produced by the processing; and

(ii)	using at least 1 other crystalline silica control measure; and

(iii)	ensuring each worker at the workplace who may be exposed to airborne crystalline silica produced by the processing—

(A)	is provided with respiratory protective equipment; and

(B)	wears the respiratory protective equipment while the work is carried out.

32	New division 7A.3.3

insert

Division 7A.3.3	Regulator to be notified of particular processing of engineered stone

418I	Notification of particular processing of engineered stone

(1)	This section applies if work that involves processing engineered stone benchtops, panels or slabs is carried out—

(a)	to remove, repair or make minor modifications to installed engineered stone; or

(b)	to dispose of engineered stone, whether it is installed or not.

(2)	Before the work is carried out, a person conducting a business or undertaking carrying out, or directing or allowing a worker to carry out, the work mentioned in subsection (1) must give the regulator written notice—

(a)	stating the work being carried out; and

(b)	describing the type of work being carried out; and

(c)	stating the frequency and duration of the work; and

(d)	stating any other information in relation to the work required by the regulator.

Maximum penalty:

(a)	in the case of an individual—$5 000; or

(b)	in the case of a body corporate—$25 000.

Note	Strict liability applies to each physical element of each offence under this regulation, unless otherwise stated (see s 6A).

(3)	Subsection (2) does not apply if—

(a)	the person conducting the business or undertaking does not know, and could not reasonably be expected to know, before the work is carried out that the work involves processing engineered stone benchtops, panels or slabs; and

(b)	as soon as practicable after the person conducting the business or undertaking becomes aware that the work involves processing engineered stone benchtops, panels or slabs, the person gives the regulator written notice under subsection (2) in relation to the work.

(4)	If the regulator receives written notice under subsections (2) or (3), the regulator must give the person conducting the business or undertaking acknowledgment of receipt of the notice.

Note 1	If a form is approved under the Act, s 277 for this provision, the form must be used.

Note 2	A fee may be determined under the Act, s 278 for this provision.

Note 3	It is an offence to make a false or misleading statement, give false or misleading information or produce a false or misleading document (see Criminal Code, pt 3.4).

418J	Duty to keep notice given under div 7A.3.3

A person conducting a business or undertaking who gives the regulator written notice under this division must, for a period of 5 years beginning on the day the notice is given to the regulator—

(a)	keep a copy of the notice; and

(b)	ensure a copy of the notice is readily accessible; and

(c)	allow a person to access a copy of the notice upon request.

Maximum penalty:

(a)	in the case of an individual—$5 000; or

(b)	in the case of a body corporate—$25 000.

33	New division 11.2.5

insert

Division 11.2.5	Crystalline silica material

698AA	Definitions—div 11.2.5

In this division:

crystalline silica material—see section 418A (1).

engineered stone—see section 418A (1).

698AB	Exemptions under corresponding WHS laws—work involving engineered stone

A person conducting a business or undertaking is exempt from compliance with section 418F if the work involves a type of engineered stone that is the subject of an exemption—

(a)	granted in relation to the type of engineered stone under a corresponding WHS law; and

(b)	notified to the regulator by a corresponding regulator.

34	Dictionary, new definition of crystalline silica

insert

crystalline silica, for chapter 7A (Crystalline silica)—see section 418A (1).

35	Dictionary, definition of cut

omit

36	Dictionary, new definition of mechanical method

insert

mechanical method, for chapter 7A (Crystalline silica)—see section 418A (1).

37	Dictionary, definition of mechanical process

omit

38	Dictionary, new definitions

insert

porcelain product, for chapter 7A (Crystalline silica)—see section 418A (1).

process, for chapter 7A (Crystalline silica)—see section 418A (1).

processing, in relation to engineered stone, for part 7A.3 (Engineered stone)—see section 418E.

sintered stone, for chapter 7A (Crystalline silica)—see section 418A (1).

stone-substitute material, for chapter 7A (Crystalline silica)—see section 418A (1).

39	Further amendments, dictionary, definitions

omit

section 418A

substitute

section 418A (1)

in

	dictionary, definitions of

airborne crystalline silica

engineered stone

high risk crystalline silica work

respiratory protective equipment

wet dust suppression method

Schedule 1	Other amendments

(see s 3)

[1.1]	Section 418F, penalty

substitute

Maximum penalty:  tier E monetary penalty.

[1.2]	Section 418I (2), penalty

substitute

Maximum penalty:  tier G monetary penalty.

[1.3]	Section 418J, penalty

substitute

Maximum penalty:  tier G monetary penalty.

Endnotes

1	Notification

Notified under the Legislation Act on 28 June 2024.

2	Republications of amended laws

For the latest republication of amended laws, see www.legislation.act.gov.au.

© Australian Capital Territory 2024